| Sanitation/Biosecurity (Element 2.1a&b) - Control Measures |  |  |  |
| --- | --- | --- | --- |
|  |  | SFCR 50(1&2) |  |
| Responsible Person (who): | When to apply: | Where to apply: | Record |
|  | Daily During Operation | Prawn Handling/packing Areas, Equipment and Utensils | Sanitation, Hygiene and Chemical Use Monitoring Form- in During-Season Log |
| General Cleaning Procedures: The ‘responsible person’, ensures that receiving areas, equipment, containers and utensils are cleaned and disinfected at least once a day during operation. The ‘responsible person’ either completes the tasks personally, or directs a properly trained employees to complete the tasks. Before you begin cleaning/sanitation procedures, you must prevent the chemicals, or rinse water from contacting all packaging, input materials or processed goods by: i. removing the items from the area or ii. covering the items with a clean, sound divider like a tarp or plastic sheet OR iii. isolating the items using a divider. The 5-Step Cleaning/Sanitation procedure for equipment, utensils and surfaces used in the handling of shrimp/prawn: Slime, blood and scales that accumulate should be cleaned off before they dry up and are difficult to remove To clean/sanitize the fish handling and packing areas of the vessel, equipment and utensils follow these basic steps: - Pre-rinse the surfaces and equipment with a high pressure jet of cold water to remove excess slime, blood and scales. - Scrub the surfaces and equipment using a stiff brush or a high pressure cleaner in combination with an acceptable detergent (mixed and applied as per Element 2.3 Chemical Use) for appropriate time and temperature - Rinse with cold water to remove all traces of detergent - Sanitize using cold water mixed with mild bleach solution or other bleach solution (see above- mix and apply as per Element 2.3 Chemical Use) for appropriate time and temperature - Rinse with cold water to remove disinfectant Containers, utensils, penboards and shelfboards: i. Air dried before they are stacked and stored |  |  |  |

Cleaning Equipment: Equipment & material used to clean and disinfect provided must be in adequate quantities and conveniently located in designated area off the floor, approved and labelled for use. Include a list of cleaning equipment in the PRE-SEASON EQUIPMENT INSPECTION FORM (element 4.1) (in the Pre-Season Log) and keep approval letters on file. ____________________________________________________________________________________ Sanitation of Live tank holding systems: After Each delivery – - the entire live holding tank and chilling system must be thoroughly cleaned and sanitized with a bleach solution consisting of approximately a capful of bleach per gallon of approved source of potable fresh water (follow manufacturers directions). - At least 20 minutes contact time is needed for the solution to properly sanitize the lines. - If systems cannot be drained completely overnight, the bleach solution should remain in the lines and be flushed out thoroughly in the morning before takin on fresh seawater for the days fishing. - This is especially crucial for parts of the chilling system located in warm engine compartments where the water inside the system could warm up and cause bacteria to grow.

| Sanitation/Biosecurity (Element 2.1a)b) - Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| Monitor: | What: | How: | Frequency: | Record |
|  | Sanitation procedures and effectiveness | See below | Daily While in operation | Sanitation, Hygiene and Chemical Use Monitoring Form- in During-Season Log |
| How to Monitor Cleaning/Sanitation Effectiveness (SFCA Reg 50(1&2): Daily while in operation, the ‘monitor’ will inspect the processing and storage areas of the vessel. Procedures: A) i. Ensure that the written sanitation procedures and schedules are readily available. ii. Ensure that necessary cleaning or sanitizing and/or biosecurity/biocontainment equipment is available, functioning as intended, clean, appropriately stored and labeled iii. Ensure that chemicals are available and appropriately labelled B) Visually inspect and ensure that: i. Contact surfaces are free from contamination (e.g., the accumulation of dust, dirt, food residue and other debris). ii. Visually verify that cleaning and/or biosecurity/biocontainment procedures are effectively removing dirt, debris or any other contamination risk. iii. Look for evidence of pest or disease, dirt, debris and residual material on direct commodity contact surfaces, including containers and utensils. C) Monitoring activities are recorded on the Sanitation, Hygiene and Chemical Use Monitoring Form, initialed and dated. D) Ensure that corrective actions are made as required (see Appendix 1) and recorded appropriately. |  |  |  |  |

| Sanitation/Biosecurity (Element 2.1a)b) - Verification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Verifier: | What: | How: | Frequency: | Record |
|  | Sanitation procedures and effectiveness | See below | Each trip or at least weekly | Sanitation, Hygiene and Chemical Use Monitoring Form- in During-Season Log |
| How to Verify Cleaning/Sanitation Effectiveness (SFCA Reg 50(1&2): Once per trip or at least weekly if trips extend past a week, the ‘verifier’ will review the ‘Sanitation, Hygiene and Chemical Use Monitoring Form’ Ensure: - All monitoring procedures are complete - Observes vessel, storage areas, equipment and utensils for signs of contamination (eg. Dirt, debris, chemical residue) - Interview (as necessary) employees responsible performing the activity to ensure that procedures are taking place as written and training has been effective. - Take samples to determine effectiveness and compliance, if necessary, to confirm observations. - Corrective Actions are taken if required |  |  |  |  |