| CHEMICALS – Control Measures |  |  |  |
| --- | --- | --- | --- |
| Chemical Selection (Element: 2.3a) |  | SFCR 52 (b) |  |
| Responsible Person: | When to apply: |  | Record(s) |
|  | Prior to beginning of Fishing Season |  | Approval letters |
| Prior to the Beginning of the Season: How to Select non-food Chemicals (other than pest control): Use one or more of the following selection methods: - Select chemicals from the CFIA List (on CFIA web site) Canadian Food Inspection Agency - Guidelines for Submissions - Reference Listing of Accepted Construction Materials, Packaging Materials and Non-Food Chemical Products and/or - Select chemicals from the NSF approval web site (or other certification agencies) - Have on file a Letter of Acceptance (LOA) previously provided by the CFIA; and/or - Have on file a Letter of No Objection (LONO) from Health Canada for the product used. Note that Health Canada will continue to assess packaging materials and non-food chemicals (not construction materials) on a voluntary basis if requested by industry; and/or - Have on file a Letter of Guarantee (LOG) from the supplier of the chemicals. For pest control chemicals: - Select items from Health Canada’s ‘pesticide public registry web site. - In the site -Refer to ‘product information’ and ensure that the products application outcome is listed as ‘registered’ Once an approval has been found: - all documentation should be filed. - Note the addition of the chemical in the amendment log How to Store Chemicals: - Prawn preservative stored away from non-food chemicals - stored in a designated, appropriate, weather proof location, clean and good repair, well ventilated - conditions must be as recommended by manufacturer (eg. temperature, humidity, light) if applicable - no unnecessary chemicals shall be stored in the processing area - not stored near supplies or materials (eg product, packaging, labels) that could contaminate food - the containers must be properly labelled, clearly identifying the product and its use. If the manufacturers labels are absent or inadequate, affix a labels that clearly identify the chemical |  |  |  |

| Chemical Selection (Element 2.3) - Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| Monitor: | What: | How: | Frequency: | Record |
|  | That all chemicals are appropriately and stored | Follow the steps in the 'Pre-Season Chemical Inspection Form' | Prior to beginning of Fishing Season | Pre-Season Chemical Inspection Form |
| How to Monitor The ‘monitor’, prior to the start of the fishing season ensures that the chemicals are appropriately selected and stored by: - Following the points on the ‘Pre-season Chemical Inspection Form’ - Identifying any deficiencies and initiating ‘Corrective Actions’ - Signing and dating the ‘Pre-season Chemical Inspection Form’ |  |  |  |  |

| Chemical Selection (Element 2.3) - Verification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Verifier: | What: | How: | Frequency: | Record |
|  | Completion and effectiveness of monitoring procedures | Review all points on the 'Pre-Season Chemical Inspection Form' | Prior to beginning of Fishing Season | Pre-Season Chemical Inspection Form |
| How to Verify The ‘Verifier’, prior to the start of the fishing season ensures that the chemicals selction and storage monitoring procedures are complete by: - Reviewing all points on the ‘Pre-season Chemical Inspection Form’ - Review any corrective actions that were initiated during monitoring - Identifying any additional deficiencies and initiating ‘Corrective Actions’ - Signing and dating the ‘Pre-season Chemical Inspection Form’ |  |  |  |  |

| CHEMICAL USE INSTRUCTIONS (Element 2.3b) – CONTROL MEASURES |  |  |  |
| --- | --- | --- | --- |
| Chemical Use Instructions (Element: 2.3b) |  | SFCR 52 (b) |  |
| Responsible Person: | When to apply: | Record | Monitor |
|  | Each time chemicals are mixed | Daily Sanitation, Hygiene and Chemical Use Monitoring Form |  |
| Using and applying chemicals: - Chemicals must be mixed and applied according to the manufacturer’s instructions, by a trained individual - Instructions must be readily available to the individuals responsible for using them- either on the label, or a written document - Must be handled and used in a manner that does not present a risk of contamination of a food (see 5-Step cleaning/sanitation Procedure, Element 2.1 a&b) Sanitation/Biosecurity/Biocontainment) Specific Mixing Instructions: - Mixing Detergent: Used as directed on label - Mix Sanitizer:- mix as per manufacturer’s instructions or for bleach as below: Household bleach (5% bleach) 200ppm bleach solution (as recommended by BC Interior Health for food contact surfaces) How to mix 200ppm bleach solution (from household 5% bleach) Per Liter; Per Bucket; Per Fish Tote. 5 ml bleach per liter of water; 100 ml bleach per 20 liter (5 gallon) bucket of water; 5 liters bleach per 1000l fish tote of water. *use half the amount of bleach if using 10% industrial bleach Mixing Prawn Chemicals: - Ensure that the prawn chemicals have the mixing instructions attached to the bags (or readily available) - Read and follow ALL directions as written each time that the chemicals are mixed. - Use any safety equipment and take any safety precautions recommended |

| Chemical Use (Element 2.3) - Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| Monitor: | What: | How: | Frequency: | Record |
|  | Chemical use | Visually monitor behaviour. | Daily while in operation | Daily Sanitation, Hygiene and Chemical Use Monitoring Form- See During Season Log |
| How to Monitor The ‘monitor’, with reference to the Daily Sanitation, Hygiene and Chemical Use Monitoring Form ensures chemicals are: - appropriately stored & labeled - instructions are readily available - correctly applied by a trained individual, in a manner that does not risk contamination of packaging or product - indicated on the form if the procedures are effective - indicates if any corrective actions are needed (see Appendix 1). - initials and dates the form |  |  |  |  |

| Chemical Use (Element 2.3) - Verification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Verifier: | What: | How: | Frequency: | Record |
|  | Effectiveness of chemical use monitoring | Reviews the Daily Sanitation, Hygiene and Chemical Use Monitoring Form | Once per trip or at least weekly | Daily Sanitation, Hygiene and Chemical Use Monitoring Form |
| How to Verify The ‘Verifier’, Once per trip, or at least once per week if a trip extends beyond a week ensures that monitoring activities are being completed and effective by: - reviewing the Daily Sanitation, Hygiene and Chemical Use Monitoring Form to ensures that all elements have been completed - spot check chemicals for appropriate labelling (processing and storage areas) - ensure instructions for use are readily available - verify that only trained individuals are using and applying the chemicals - record verification activity, initialed and dated on the Daily Sanitation, Hygiene and Chemical Use Monitoring Form - Corrective Actions are to be recorded and filed (see Appendix 1). |  |  |  |  |