| Water (Element: 5.3) - Control Measures |  |  |
| --- | --- | --- |
|  | SFCR 70 (1,2,3); 71 (1 & 2) |  |
| Responsible Person (who): | When to apply: | Record |
|  | when water is sourced | PRE-SEASON VESSEL WATER SUPPLY INSPECTION FORM |

Water Supply:

Ensure the water used to wash the fish or clean and sanitize surfaces and equipment or make ice does not present a source of contamination.

- Water must be available in a quantity sufficient to meet the needs of your operation

- There is no cross-connections between safe and unsafe water supplies- evaluate pipes, and drains to ensure there is not cross contamination

- hoses, taps or other delivery systems fitted with backflow preventers on fixtures or vacuum breakers

- Suitable drawing (complete the ‘Water Source and Water Systems diagram in the ‘Pre-Season Log’) showing water supply and  waste water systems ( sources of supply, intake locations, piping runs, treatment systems employed, location of water-sampling valves for the taking of water samples before and after its treatment and the outfall or sewage hook-up locations.)

Municipal Freshwater:

- Ensure there is no ‘water advisory’ from the municipality (usually posted on the wharf faucets) before filling reservoir-

- Each of the British Columbia Health Authorities list water Sampling results on line which should be referenced as needed:

  - Northern Health. https://www.northernhealth.ca/services/environmental-health/drinking-water/water-sampling-and-results

  - Interior Health. https://www.interiorhealth.ca/YourEnvironment/DrinkingWater/Pages/WaterSamples.aspx

  - Island Health (Vancouver Island) https://www.healthspace.ca/clients/viha/viha_website.nsf/Water-Samples-Main?OpenView

  - Vancouver Coastal Health. http://www.vch.ca/public-health/environmental-health-inspections/drinking-water

  - Fraser Health. https://www.fraserhealth.ca/health-topics-a-to-z/drinking-water#.Xay2UZJKhqN

Seawater:

- Ensure that it is not drawn from around harbours or docks and in close vicinity to towns, villages, industrial plants, fish processing plants, freezer factory ships, sewage outfalls, discharge pipes, fuel or oil slicks.

- Reference historic final end product result tests supplied by buyers (see Appendix 4) as a way to assess water acceptability.

- Water intakes on the vessel as far from vessel drainage and discharge as possible

- Do not dispose of sewage or fish waste in areas where you source water

Ground water- private source

- Head well securely covered

- A sample for total coliforms is tested within one month prior its use

- Sampled from the source (outlet where you draw the water from). Refer to CFIA web site ‘Preventive Controls Food Businesses- Sampling procedures for how to draw the water

- The result must be =  total coliforms undetectable in 100ml (kept on file)

Treated, potentially contaminated water (eg. Filtration, ultraviolet, reclaimed water):

  - The treatment system is operated, cleaned and maintained as per manufacturers instructions

  - Sampled monthly while in operation

  - Sampled from the source (outlet where you draw the water from). Refer to CFIA web site ‘Preventive Controls Food Businesses- Sampling procedures for how to draw the water

  - The result must be =  total coliforms undetectable in 100ml (kept on file)

□Other (detail any other water control systems, sources or treatments):__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

| ### Water (Element: 5.3) -Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| Monitor (responsible monitoring equipment) | What: | How: | Frequency (how frequently it is checked to ensure training completed): | Record: |
|  | Source & controls for water | Ensure that water is from a low risk source | When water is sourced | 1) PRE-SEASON VESSEL WATERSUPPLY INSPEC TION FORM 2) Sanitation Hygiene and Chemical Use Form |
| How to Monitor: Prior to the Start of the Season: - Complete the PRE-SEASON VESSEL WATER SUPPLY INSPECTION FORM including the Schematic (Water source and waste water systems). Ensure water intakes on the vessel as far from vessel drainage and discharge as possible - Identify and deficiencies and schedule repairs on the Maintenance/Repair Table as if necessary OR initiate corrective actions if other than routine maintenance - Sign and date the forms During the Season: Each time water is sourced, the monitor: - Ensure that there are no water advisories for municipal fresh water - Seawater - is is not drawn from around harbours or docks and in close vicinity to towns, villages, industrial plants, fish processing plants, freezer factory ships, sewage outfalls, discharge pipes - Do not dispose of sewage or fish waste in areas where you source water - Daily monitoring is recorded on the Sanitation Hygiene and Chemical Use Form - If a problem arises refer to ‘Corrective Action Procedures’ (Appendix 1), corrective actions must be taken and recorded on a Corrective Action Form. |  |  |  |  |

| ### Water (Element: 5.3) - Verification |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Verifier (responsible monitoring equipment) | What: |  | How: | Frequency (how frequently it is checked to ensure training completed): | Record: |
|  |  | sourcing of water | Review the records | 1) Once prior to the start of the season 2) Each Trip | 1) PRE-SEASON VESSEL WATER SUPPLY INSPECTION FORM 2) Sanitation Hygiene and Chemical Use Form |
| How to Verify: Prior to the Start of the Season: - Review the PRE-SEASON VESSEL WATER SUPPLY INSPECTION FORM including the Schematic (Water source and waste water systems) - Review Corrective Actions that arose from monitoring - Identify any deficiencies and schedule repairs on the Maintenance/Repair Table as if necessary OR initiate corrective actions - Sign and date the forms During the Season: Each trip or at least weekly: - Review the Sanitation Hygiene and Chemical Use Form to ensure that all monitoring steps are completed - Review Corrective Actions that arose from monitoring - Verify by interviewing employees that - Freshwater was from an approved source - Seawater - is is not drawn from around harbours or docks and in close vicinity to towns, villages, industrial plants, fish processing plants, freezer factory ships, sewage outfalls, discharge pipes - Do not dispose of sewage or fish waste in areas where you source water - Verification testing of final end product (see Element 1.1a) must be completed. - Sign and date the Sanitation Hygiene and Chemical Use Form # Corrective Actions-If a problem arises refer to ‘Corrective Action Procedures’ (Appendix 1), corrective actions must be taken and recorded on a Corrective Action Form. If final end testing exceeds the standard for E. coli [(5 samples units from a lot – 2 samples may exceed 4 mpn/g, but none may exceed 40mpn/g; (from Bacteriological guidelines for fish and fish products (end product))] undertake an investigation to identify the harvest area of the product, the source of the water used for processing and review sanitation and hygiene records/procedures to determine the source of the bacteria. |  |  |  |  |  |