| Waste Disposal (Element: 5.4) - Control Measures |  |  |  |
| --- | --- | --- | --- |
|  |  | SFCR 66 (1&2) |  |
| Responsible Person: | When to apply: | Where to apply: | Record |
|  | 1) prior to season (equipment) & 2) daily during operations | Processing Area | 1) Pre-Season Waste Disposal Inspection Form 2) Sanitation, Hygiene and Chemical Use Monitoring Form |

Drainage and Sewage: - The drainage and sewage systems must accommodate the volume and type of effluent produced in establishment, allowing for constant fluid removal with no pooling - Drainage and sewage systems are equipped with traps and vents to prevent backflow Waste Storage: Waste storage areas and containers are identified, of suitable capacity, secured and cleaned to avoid attracting pests. Apply ‘Element 4.1) Equipment Design and Maintenance’ to select waste bins. a) approved construction of containers &. facilities. b) provided with fitted lids. a) of approved material. b) properly maintained. Frequency and manner of waste disposal (SFCR 66(2): - Remove waste and contaminated materials at least once per trip or more often if necessary so that they do not overflow - Remove contaminated materials and waste from the vessel using predetermined routes (apply 5.2 b) Hygienic Flow and Separation)- Not through the processing and storage areas - Effluent or sewage lines such that they do not pass directly over or through production areas - Employees wear appropriate clothing, such as overalls, gloves and boots, when removing the contaminated materials and waste, if appropriate (apply Element 3.1 Hygiene, Biosecurity)

| ### Waste Disposal (Element 5.4)- Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| Monitor (who): | What: | How: | Frequency (how frequently it is checked to ensure training completed): | Record: |
|  | Waste disposal equipment and methods | Visually inspect equipment and waste disposal methods | 1) waste disposal systems -prior to the fishing season 2) waste disposal methods- daily during operations | 1) Pre-Season Waste Disposal Inspection Form 2) Sanitation, Hygiene and Chemical Use Monitoring Form |
| How to Monitor: Pre-Season Inspection- The Monitor, prior to the start of the fishing season, will complete the ‘Pre-season Waste Disposal Inspection Form’. Form is signed and dated when the inspections take place. - Inspect all items listed on the form, and ensure they conform with the requirements as listed on the form - make notes of any deficiencies or general findings - If items need to be repaired/replaced etc., add the issue to the ‘Maintenance/Repair table’, schedule repair date and verify on completion. Keep ‘corrective action’ form on file (see Appendix 1). During-Season Inspection- The Monitor, on each operating day, will observe: - Waste disposal written instructions (Waste Disposal- Element 5.4) are available - Waste disposal containers & equipment – available, sound condition, labelled and cleaned after emptied - Emptied in appropriate manner that will not contaminate product, ingredients, packaging or water source - Initiate ‘corrective actions’ if necessary (see Appendix 1). |  |  |  |  |

| ### Waste Disposal (Element 5.4)- Verification |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Verifier (who): | What: |  | How: | Frequency (how frequently it is checked to ensure training completed): | Record: |
|  |  | Effectiveness of waste disposal monitoring | Review monitoring forms. Observe equipment, utensils and methods | 1) waste disposal systems -prior to the fishing season 2) waste disposal methods- each trip | 1) Pre-Season Waste Disposal Inspection Form 2) Sanitation, Hygiene and Chemical Use Monitoring Form |
| How to Verify: The verifier, prior to the start of the fishing season: - Reviews the ‘Pre-season Waste Disposal Inspection Form’ to ensure that all elements are completed. - Reviews ‘Corrective Actions’ that arose from monitoring - Observes the waste disposal equipment and systems to ensure they are approved - Containers and utensils labelled - The verifier signs and dates the ‘Pre-season Waste Disposal Inspection From’ and indicates if ‘corrective actions’ were needed (see Appendix 1). The verifier, once per trip during the fishing season: - Reviews the ‘sanitation, hygiene, and chemical use monitoring form’ to ensure that all elements are completed. - Reviews ‘Corrective Actions’ that arose from monitoring - Observes the waste disposal methods - Containers and utensils labelled - The verifier signs and dates the ‘sanitation, hygiene, and chemical use monitoring form’ and indicates if ‘corrective actions’ were needed (see Appendix 1). |  |  |  |  |  |