Pre-season Log

This log is designed to capture all monitoring procedures that should be completed prior to the start of the fishing season.  It is recommended that they are completed in the following order:

| Log Form | Refer to ‘Preventive Control Plan’ Element for more detail | Comment |  |
| --- | --- | --- | --- |
| PRE-SEASON TIME FOR PROCESSING PRAWNS INSPECTION FORM | Processing Prawns (Element 1.1c) | This ensures that processing times are sufficient to reduce risk of pathogen growth |  |
| Pre Season Training Form | Employee Training (Element 3.2) |  |  |
| PRE-SEASON EQUIPMENT INSPECTION FORM | Equipment design and maintenance (Element 4.1)- |  |  |
| PRE-SEASON VESSEL CONSTRUCTION INSPECTION FORM | Vessel Design Construction and Maintenance (Element: 5.2 a) |  |  |
| Hygienic Flow and Separation- diagrams | Hygienic Flow and Separation (Element: 5.2 b) |  |  |
| PRE-SEASON LIGHTING INSPECTION FORM | Lighting (Element: 5.2 c) |  |  |
| PRE-SEASON VENTILATION INSPECTION FORM | Ventilation (Element: 5.2 d) |  |  |
| PRE-SEASON EMPLOYEE FACILITIES INSPECTION FORM | Employee Facilities (Element: 5.2 e) |  |  |
| Water source diagram and PRE-SEASON VESSEL WATER SUPPLY INSPECTION FORM | Water (Element: 5.3) |  |  |
| PRE-SEASON WASTE DISPOSAL INSPECTION FORM | Waste Disposal (Element: 5.4) |  |  |
| PRE-SEASON PACKAGING CONTROLS INSPECTION FORM | - Packaging Controls (element 1.2a - Product ID- Labelling (element 1.2b) |  |  |
| PRE-SEASON CHEMICAL INSPECTION FORM | Chemical Selection (Element: 2.3) |  |  |
| PRE-SEASON CONVEYANCE INSPECTION FORM (element 6.1a) | Conveyances for Transport (Element: 6.1 a) | Complete only if you have your own vehicle for transporting product, packaging or ingredients |  |
| PRE-SEASON FREEZER AND COOLER STORAGE TEMPERATURE INSPECTION FORM | Conveyance & Storage Temperature & Humidity (Element: 6.1 b) | Do freezing tests if possible before the season, or during the season if more feasible |  |

- The ‘Maintenance/Repair Table’ is Page 29

| PRE-SEASON TIME FOR PROCESSING PRAWNS INSPECTION FORM (element 1.1c) SFCR 47 |  |  | Complete prior to the start of the season |
| --- | --- | --- | --- |
| Monitor (signature): | Date Monitored: | Verifier (signature): Check that all relevant inspection points have been completed | Date Verified: Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |
| Record the time taken at each step from trap pull until freezing: 1; Time (specify minutes or hours). 2. 3. 4. 5. 6. 7. 8. Total time of exposure to air:. Total cumulative time prawns are exposed to the air, from the time it is harvested to the time it is frozen (to limit bacterial growth) should be (Fish and Fishery Products Hazards and Controls Guidance- US DoH): - <5 hours (300 min) if air temperature is between 11 and 21°C - <2 hours (120 min) if air temperature exceeds 21°C |

| PRE-SEASON TRAINING FORM (Element 3.2)- Ensure Employees Read and Understand Required Elements (retrain each season, when corrective actions are needed or when there are changes to procedures): |  |  | Complete prior to start of the season |  |
| --- | --- | --- | --- | --- |
| Who Requires Training | Element (identify any additional training needed): | Employees Trained | Date | Trainer (initials) |
| all employees | PROCESS CONTROLS (Element 1.1a Incoming Inputs ) |  |  |  |
| employees who mix preservative | PRODUCT FORMULATION (Element 1.1b) |  |  |  |
| all employees | PROCESS CONTROLS (Element 1.1c) |  |  |  |
| employees who receive, handle and store packaging | PACKAGING CONTROLS (Element 1.2a) |  |  |  |
| employees who apply lot codes | PRODUCT IDENTIFICATION (Element 1.2b) |  |  |  |
| all employees | SANITATION (Element 2.1 a&b) |  |  |  |
| employee who monitors sanitation | SANITATION EFFECTIVENESS (Element 2.1 c) |  |  |  |
| all employees | PEST CONTROL (Element 2.2) |  |  |  |
| employees who apply and store chemicals | CHEMICAL USE & STORAGE (Element 2.3 b-e) |  |  |  |
| all employees | PERSONAL PROTECTIVE GEAR (Element 3.1 a) |  |  |  |
| all employees | PERSONAL HYGIENE (Element 3.1 b) |  |  |  |
| maintenance personnel | EQUIPMENT AND MATERIALS SELECTION (Element 4.1 a) □ Additional training certificate(s) on file if required |  |  |  |
| maintenance personnel | CALIBRATION (controlling and measuring devices)(Element 4.1b) □ Additional training certificate(s) on file if required |  |  |  |
| maintenance personnel | VESSEL CONSTRUCTION (Element 5.2) □ Additional training certificate(s) on file if required |  |  |  |
| maintenance personnel | CONVEYANCES (Element 6.1 a) □ Additional training certificate(s) on file if required |  |  |  |
| maintenance personnel | TEMPERATURE & HUMIDITY (Element 6.1 b) □ Additional training certificate(s) on file if required |  |  |  |
| all employees | LOADING/UNLOADING (Element 6.1 c) |  |  |  |
| all employees | COMMODITY STORAGE (Element 6.1 d) |  |  |  |
| Employees who keep tracking records | TRACEABILITY- Element 7.1 |  |  |  |
| Employees who facilitate recall | COMPLAINTS & RECALL (Element 7.2) |  |  |  |

*monitoring of effectiveness is completed as part of ‘During-Season Log’

Verifier:______________________    Signature:___________________      Date:___________________

Verifier Ensures that :

□- Training has been provided for all elements (as indicated above)

□- All employees requiring training are included on the form

□- Elements are being effectively executed during fishing trips (see Verifications are in the ‘During Trip Controls’ elements)

RE-TRAINING NOTES (retraining can be recorded on a ‘Corrective Action Form’:

| PRE-SEASON EQUIPMENT INSPECTION FORM (element 4.1a) SFCR 53(c)(i-iv) |  |  | Complete prior to the start of the season |
| --- | --- | --- | --- |
| Monitor (signature): | Date Monitored: | Verifier (signature): Check that all relevant inspection points have been completed | Date Verified: Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |
| Equipment and conveyances used to handle, prepare, package and/or label fish Product Contact Surfaces and Utensils (direct contact) (check all that apply and list others as necessary): □- Tables & Benches □- Chutes □- Mechanical Sorter □- Conveyors □- Washers □- Utensils (knives, scoops, shovels etc.) □- Trays □- Containers & Traps □- Receptacles □- Buckets □- Pallets □-Cleaning equipment (scrub brushes, scrapers, buckets etc) □- Other (list all others that apply): ______________________________________________________________________ ______________________________________________________________________ ______________________________________________________________________ ______________________________________________________________________ ______________________________________________________________________ Each of the above items must be: - constructed of smooth, durable, cleanable, non-corrosive, non-absorbent materials free of noxious substances (stainless steel, salt and water-resistant aluminium alloys, high density plastic or fibreglass reinforced plastic, NOT wood or enamel), withstands repeated cleaning/sanitizing - made of materials that are not affected by salt or water (galvanized metal or epoxy coated wood) - repaired, cleaned and stored in sanitary conditions prior to the season (especially traps) - approvals on file - free of pits, cracks, flakes and crevices - no unsealed ends (eg. bottom of table legs) - properly installed and maintained - accessible for maintenance, cleaning, sanitizing, inspection and disassembly (SFCR 53 (b&f)) - no internal horizontal ledges or hidden hard to- clean- areas (SFCR 53 (b)) - Stored off the floor in designated area - equipment for contaminated materials, waste identified (labelled, colour coded etc) used only for that purpose - cleaning equipment and utensils identified (eg. labelled) for purpose - Pallets may be wood if clean, in sound condition and used for holding; boxed or containerized raw material, ingredients, additives, packaging material, raw material, labels, boxed or containerized finished product. |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) (Example: Table for sorting prawns is constructed of wood. Will be replaced with water resistant aluminum. Date of replacement scheduled on Maintenance/Repair Table) |
| Mechanical Equipment Requirements (in addition to those above) (check all that apply): □- Generators □- Refrigeration/Freezing units □- Compressors □- Other (list all that apply): ___________________________________________ ___________________________________________________________________ - no lube drip (food grade lubricant recommended) - equipped with instruments to control, indicate and record any parameters that are necessary to prevent contamination, calibrated and maintained and function as intended SFCR 53 (b,d, e) - exhausts outside the vessel (SFCR 53 (b)) - provide for drainage, where required (eg condensation drip) (SFCR 53 (b) - protective shield or lids to prevent contamination (SFCR 53 (b)) - doesn't contaminate food if a part fails (SFCR 53 (b)) - inspected regularly and the equipment that is in poor condition are repaired or replaced ______________________________________________________________________ Other Mechanical equipment, fuel or waste related equipment requirements (check all that apply): Each of the items below must be: Isolated from storage areas or enclosed in a manner that prevents them from contaminating the fish □- Chain drives □- Drive-shafts and bearings □- Fuel lines □- Fuelling ports □- Fuel storage tanks □- Waste disposal lines and □- Other (list all that apply): ____________________________________________ |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) (Example: Generator exhausting into the packing area where it could contaminate product. Extension for pipe scheduled on Maintenance/Repair Table) |
| Holds for product (check all that apply): □- Water tanks □- Boxes □- Shelving □- Pens □-Other (list all that apply): ______________________________________________________________________ ______________________________________________________________________ Each of the above items must be: - made of stainless steel, corrosion resistant aluminium, high density plastic, fibreglass reinforced plastic or coated smooth wood. - approvals on file joints sealed (smooth & watertight) to prevent accumulation of contaminants. - free of pits, cracks, flakes and crevices properly installed and maintained - accessible for maintenance, cleaning, sanitizing, inspection and disassembly (SFCR 53 (b&f)) |  |  | (Example: Boxes for holding prawns heavily scoured and cannot be cleaned effectively. Replacement scheduled on Maintenance/Repair Table) |
| PRE-SEASON CALIBRATION INSPECTION FORM (element 4.1) |  |  | Complete prior to the start of the season |
| Monitor (signature): | Date Monitored: | Verifier (signature): Check that all relevant inspection points have been completed | Date Verified: Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |
| Equipment That Requires Calibration (check all that apply): - Refrigeration temperature recorders - Freezer Refrigeration recorders - Hand Held Probe thermometers - Infrared Thermometers - Other (list all that apply): __________________________________________ _____________________________________________________________________ _____________________________________________________________________ For Calibrations (check to indicate these steps were followed): - Calibrate according to the manufacturer's instructions- - ensure the instructions on file; - done in a manner that does not present a risk of contamination of the food - done by a qualified employee or a service contractor - not done in the same room and at the same time as food preparation - done at a frequency that ensures accuracy. - Record calibration results in comments section on this form (keep 3rd party service reports on file) - appropriate actions are taken when calibration issues are identified Hand Held Thermometers: Calibrated using: - manufacturers instructions or - using ice bath procedures (see control measures or the calibration log) - Must be calibrated weekly during the fishing season |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) |

| PRE-SEASON VESSEL CONSTRUCTION INSPECTION FORM (element 5.2a) |  |  | Complete prior to the start of the season |
| --- | --- | --- | --- |
| Monitor (signature): | Date Monitored: | Verifier (signature): Check that all relevant inspection points have been completed | Date Verified: Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |
| Walls and floors in the areas where you handle, prepare, package, label and store prawns are (check all that apply): □- Processing area □- Prawn Packing area □- Label application area □- Cooler area □- Freezer area □- Other (list any addition prawn handling/processing areas) ______________________________________________________________________ ______________________________________________________________________ ______________________________________________________________________ ______________________________________________________________________ The floors and walls in the areas listed must be: - smooth, non absorbent, impervious to moisture, cleanable, free of noxious constituents, durable (resists cracks and chipping), resistant to deterioration, can withstand repeated cleaning/sanitizing (SFCR 57(b)(iii) and (iv) & (c)(iii,iv,v)) - Construction material approvals on file (see Vessel Design-Element 5.2a) - In good repair (free of exposed wood, pits, cracks) - floors proper slope to drain (or no risk of liquid accumulation) (SFCR 57(b)(iv)) - walls, floors and ceiling joints are sealed (prevent contaminant accumulation) (SFCR 57(c)(iii) and (iv)) - Light coloured walls recommended - Storage areas have joints watertight (prevent entry of contaminants) - Storage areas have edge on opening to hold raised (prevent contaminant entry from the deck) |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) (Example: Floors in processing area warn and showing bare wood. Resurfacing scheduled on Maintenance/Repair Table) |
| Drains in the areas where you handle, prepare, package, label and store prawns are (check all that apply): □- Processing area □- Prawn Packing area □- Label application area □- Cooler area □- Freezing area □- Frozen Storage area □- Other (list any addition prawn handling/processing areas) ______________________________________________________________________ ______________________________________________________________________ ______________________________________________________________________ Drains in the areas listed must be: - drainage system on the vessel is located as far away from the water intakes as possible, closer to the stern - Bilge Wells- have drainage system to prevent overflow of bilge water - readily accessible for maintenance, cleaning/sanitizing (SFCR 57 (a)) - sufficient capacity (no pooling or backup) (SFCR 57 (d), - storage areas have a drainage system that removes the ice melt, blood and slime - smooth & impervious, durable (resists chipping/cracking), can withstand repeated cleaning/sanitizing (SFCR 57(c)(iii) and (iv)) - covers or grates in place. are non-corrodible, removable & pest proof - rodentproof - equipped with check valve and traps if connected to sewer line - carries effluent away from processing area |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) |
| NOTE- Vessels constructed prior to 1982: -wood surfaces are used only if they are coated with a durable, light coloured paint or coating and free from cracks and crevices - wooden and steel holds, fish kids, checkers and large-holding containers or pens are lined to cover cracks, crevices and gouges |  |  |  |
| Ceilings in the areas where you handle, prepare, package, label and store prawns are (check all that apply): □- Processing area □- Prawn Packing area □- Label application area □- Cooler area □- Freezing area □- Frozen Storage area □- Other (list any additional prawn handling/processing areas) ______________________________________________________________________ ______________________________________________________________________ ______________________________________________________________________ ______________________________________________________________________ Ceilings in the areas listed must be: - smooth, durable (resists cracks and chipping), non absorb, impervious to moisture, non toxic (SFCR 57(b)(iii) and (iv); (c)(iv)) - light-colored (recommended) (FIR Sch 1(6) ) - can withstand repeated cleaning/sanitizing (SFCR 57(c)(iii) and (iv) ) - acceptable height (9 ft recommended) (FIR Sch 1(6) - only necessary ledges, pipes, etc. over work surfaces (FIR Sch 1(7) ) - overhead fixtures- don’t accumulate dirt, condensation, mould, or shed particles/liquids/lubricants (FIR Sch 1(7) ) - fixtures flush mounted to the ceiling and labelled for purpose (FIR Sch 1(7) ) |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) (Example: Ceilings in freezer have flaking paint. Painting with approved paint scheduled on Maintenance/Repair Table) |
| Doors in the areas where you handle, prepare, package, label and store prawns are (check all that apply): □- Processing area □- Prawn Packing area □- Label application area □- Cooler area □- Freezing area □- Frozen Storage area □- Other (list any addition prawn handling/processing areas) Doors in the areas listed must be: - smooth, durable (resists cracks and chipping), non absorb, impervious to moisture, non toxic (SFCR 57(b)(iii) and (iv); (c)(iii&iv)) - light coloured (FIR Sch 1(9) ) - can withstand repeated cleaning/sanitizing, (SFCR 57(c)(iii) and (iv) ) - kept closed to prevent pest entrance (signs recommended) (SFCR 57(b)(ii); FIR Sch II (8) ) - close fitting seals – no visible light around seals (FIR Sch 1(9) ) |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) |
| Windows in the areas where you handle, prepare, package, label and store prawns are (check all that apply): □- Processing area □- Prawn Packing area □- Label application area □- Cooler area □- Freezing area □- Frozen Storage area □- Other (list any addition prawn handling/processing areas) ______________________________________________________________________ ______________________________________________________________________ ______________________________________________________________________ ______________________________________________________________________ Windows in the areas listed must be (check as they are inspected): - openable windows windows have close-fitting insect/animal proof screens with sealed frames (SFCR 51(1) & 57(b)(ii); FIR Sch 1(8)) - smooth, non-absorbant, impervious to moisture, durable, light coloured, (unless location poses no risk to product) (SFCR 57(b)(iii) ) - windowsills sloped inward (FIR Sch 1(8) ) |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) . |

---------------------------EXAMPLE------------------------------------

Hygienic Flow and Separation (Element 5.2b)- Complete prior to the start of the season

Movement of Prawns (draw and label the movement of prawns)

1) Pull traps

11) Remove for shipping

6) Drip Drain

5) Chemical Dip

4) Purge Tanks

3) Baskets

8) Freeze

7) Pack in boxes

2) Sort /Cull

Top deck

11) Remove for shipping

10) Frozen Storage

9) Pack to Master

8) Freeze

Below deck

Monitor:______________________ (confirm accuracy of movements)

Date:__________________________

Verifier:_______________________ (confirm accuracy of movements)

Date:___________________________

Hygienic Flow and Separation (Element 5.2b)- Complete prior to the start of the season

Movement of Prawns (draw and label the movement of prawns)

Top deck

Below deck

Monitor:______________________ (confirm accuracy of movements)

Date:__________________________

Verifier:_______________________ (confirm accuracy of movements)

Date:___________________________

Hygienic Flow and Separation (Element 5.2b)- Complete prior to the start of the season

Movement of Chemicals (draw and label the movement & storage of chemicals including non-food (cleaners, sanitizers etc & prawn preservative)

Top deck

Below deck

Monitor:______________________ (confirm accuracy of movements)

Date:__________________________

Verifier:_______________________ (confirm accuracy of movements)

Date:___________________________

Hygienic Flow and Separation (Element 5.2b)- Complete prior to the start of the season

Movement of Packaging (draw and label the movement of packaging)

Top deck

Below deck

Monitor:______________________ (confirm accuracy of movements)

Date:__________________________

Verifier:_______________________ (confirm accuracy of movements)

Date:___________________________

Hygienic Flow and Separation (Element 5.2b)- Complete prior to the start of the season

Movement of Waste (draw and label the movement of waste)

(indicate water intake and drainage outflow, trash and offal storage and removal)

Top deck

Below deck

Monitor:______________________ (confirm accuracy of movements)

Date:__________________________

Verifier:_______________________ (confirm accuracy of movements)

Date:___________________________

Hygienic Flow and Separation (Element 5.2b)- Complete prior to the start of the season

Return of Rejected or Recalled product (draw and label the movement and identify the storage location)

Top deck

Below deck

Monitor:______________________ (confirm accuracy of movements)

Date:__________________________

Verifier:_______________________ (confirm accuracy of movements)

Date:___________________________

| PRE-SEASON LIGHTING INSPECTION FORM (element 5.2c) |  |  | Complete prior to the start of the season |
| --- | --- | --- | --- |
| Monitor (signature): | Date Completed: | Verifier (signature): Check that all relevant inspection points have been completed | Date Verified: Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |
| LIGHTING (Element 5.2c) Lighting) SCFR 63(1); CFIA-Lighting in an establishment guidance Lighting in the areas where you handle, prepare, package, label and store prawns are (check all that apply): □- Processing/Sorting/Inspecting area □- Prawn Packing area □- Label application area □- Cooler area □- Freezing area □- Frozen Storage area □- Packaging Storage □- Preservative Storage □- Other (list any addition prawn handling/processing areas) ___________________________________________________________________________ ___________________________________________________________________________ ___________________________________________________________________________ ___________________________________________________________________________ ___________________________________________________________________________ Light Fixtures must be: - light fixtures shielded with safety covers to contain broken material or light fixtures constructed from shatter-resistant materials or - shatterproof bulbs used (SFCR 63(2b);) (approval letter on file) - designed to facilitate cleaning (FIR Sch I (16) ) - can withstand repeated cleaning/sanitizing (SFCR 63(2a);) - lights working Acceptable light levels (natural or artificial): - 540 lux (50.2 foot candles) inspection areas & grading station - 220 lux (20.4 foot candles) processing & packaging areas - 110 (10.2 foot candles) storage areas Light levels can be checked with a telephone app. Set the phone at level where the work is being done. |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) (Example: Lighting in packaging storage area is <10.2 foot candles due to burned out bulbs. Replacement scheduled on Maintenance/Repair Table) |

| PRE-SEASON VENTILATION INSPECTION FORM (element 5.2d) |  |  | Complete prior to the start of the season |
| --- | --- | --- | --- |
| Monitor (signature): | Date Completed: | Verifier (signature): Check that all relevant inspection points have been completed | Date Verified: Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |
| VENTILATION (Element 5.2d Ventilation) Ventilation: list the areas where you handle, prepare, package, label and store prawns (check all that apply): □- Processing area □- Prawn Packing area □- Label application area □- Cooler area □- Freezing area □- Frozen Storage area □- Other (list any addition prawn handling/processing areas) ______________________________________________________________________ ______________________________________________________________________ ______________________________________________________________________ ______________________________________________________________________ Ventilation in the areas listed: - May be natural or mechanical - equipment approvals up to date for mechanical systems - prevents accumulation of odor, smoke, heat, steam, humidity, dust (SFCR 64 (a); FIR Sch I (17)) - inhibits condensation (FIR Sch I (17)) - intakes do not intake contaminated air (screens or filters if necessary) (SFCR 64 (a); FIR Sch I (17)) - intakes have non-corrodable, pest proof screens & close fitting seals (SFCR 51(1); FIR Sch I (17)) - accessible for cleaning and maintenance (can be disassembled) (SFCR 64 (b)) - can withstand repeated cleaning/sanitizing (SFCR 64 (c)) - functioning as intended (SFCR 64 (d)) - cleaned (SFCR 64 (d)) |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) (Example: Condensation in the packaging storage room. Ventilation system will be installed. Scheduled on Maintenance/Repair Table) |

| PRE-SEASON EMPLOYEE FACILITIES INSPECTION FORM (element 5.2e) |  |  | Complete prior to the start of the season |
| --- | --- | --- | --- |
| Monitor (signature): | Date Monitored: | Verifier (signature): Check that all relevant inspection points have been completed | Date Verified: Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |
| Employee Facilities (Element 5.2e Employee Facilities) SFCR 67 Lavatories □- one marine-type flush toilet and one washbasin is available for every ten crew members or □- dedicated human waste containment receptacles that are: □- impervious and cleanable materials designed to prevent spillage □- tight-fitting lids □- used only for the purpose intended □- secured and in an appropriate location to prevent contamination of the fish due to spillage or leakage □- emptied into an approved sewage disposal system □- cleaned after they are emptied □- designed so that toilet areas do not lead directly into processing areas; and (SFCR 67(3); FIR Sch I (22)) □- floor drains to prevent overflow of water or sewage □- adequately ventilated □- Properly constructed (floors, walls ceilings) □- Handwash and toilet facilities maintained in good operating order, equipped with single-service towels and toilet tissue, effluent and sewage shall be disposed of in accordance with local ordinances □- single service soap □- Toilets facilities maintained in good operating order cleaned and disinfected as often as needed (checked daily) □- hand washing notices posted (SFCR 67(1)) |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) (Example: Faucets on Handwash sink not functioning.. Replacement scheduled on Maintenance/Repair Table) |
| Hand Cleaning and Sanitizing Stations □- Washbasins equipped with non-hand-operated taps. (FIR Sch I(23)) □- warm water (comfortable to hands, softens residues, lathers soap) (SFCR 67(2) ) □- Single Service soap (SFCR 67(2); FIR Sch II(12) ) □- water pressure sufficient to remove dirt/contaminants (SFCR 67(2)) □- Washbasins 61 cm wide (recommended) (FIR Sch I(23)) □- Washbasins, in adequate quantities (1 basin/10 employees), conveniently located and visible from process area (FIR Sch I(23)) □- equipped with single service paper towels □- are readily accessible and close to the fish handling areas □- are equipped with clean running water, liquid or powdered soap, single service towels □- are maintained in a clean and sanitary condition □- can accommodate the number of employees on the vessel, for example, one marine-type flush toilet and one washbasin is available for every ten crew members □- hand washing notices posted SFCR 67(1) □- Equipped with waste receptacles SFCR 67(2) |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) |
| Changing facilities, shower, break rooms □- are appropriate to the number of persons using them; SFCR 67(1) □- readily accessible SFCR 67(1) □- can withstand repeated cleaning/sanitizing. SFCR 67(1) □- have storage facilities for street wear SFCR 67(1) |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) |

Water (Element: 5.3)

Water source and waste water systems - Complete prior to the start of the season

Show water supply and  waste water systems ( sources of supply, intake locations, piping runs, treatment systems employed, location of water-sampling valves for the taking of water samples before and after its treatment and the outfall or sewage hook-up locations.). Ensure water intakes on the vessel as far from vessel drainage and discharge as possible.

Top deck

Below deck

Monitor:______________________ (confirm accuracy of movements)

Date:__________________________

Verifier:_______________________ (confirm accuracy of movements)

Date:___________________________

| PRE-SEASON VESSEL WATER SUPPLY INSPECTION FORM (element 5.3) |  |  | Complete prior to the start of the season |
| --- | --- | --- | --- |
| Monitor (signature): | Date Monitored: | Verifier (signature): Check that all relevant inspection points have been completed | Date Verified: Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |

| Water Supply: Ensure the water used to wash the fish or clean and sanitize surfaces and equipment or make ice does not present a source of contamination. □ Water must be available in a quantity sufficient to meet the needs of your operation □ There is no cross-connections between safe and unsafe water supplies- evaluate pipes, and drains to ensure there is not cross contamination □ hoses, taps or other delivery systems fitted with backflow preventers on fixtures or vacuum breakers □ Suitable drawing (blueprint) showing water supply and waste water systems ( sources of supply, intake locations, piping runs, treatment systems employed, location of water-sampling valves for the taking of water samples before and after its treatment (if treated) and the outfall or sewage hook-up locations.) Check all types of water sources that apply: □ Municipal Freshwater: □ Ensure there is no ‘water advisory’ from the municipality before filling reservoir-typically posted on faucets at the wharf (pre-season and during season)- may be accessed via internet as well from health authorities (see element 5.3) □ Seawater: □ Ensure that it is not drawn from around harbours or docks and in close vicinity to towns, villages, industrial plants, fish processing plants, freezer factory ships, sewage outfalls, discharge pipes (review Appendix 4 for potential source locations) □ Water intakes as far from vessel drainage and discharge as possible □ Do not dispose of sewage or fish waste in areas where you source water □ Ground water- private source □ Head well securely covered □ A sample for total coliforms is tested within one month prior its use □ Sampled from the source (outlet where you draw the water from). Refer to CFIA web site ‘Preventive Controls Food Businesses- Sampling procedures for how to draw the water □ The result must be = total coliforms undetectable in 100ml (kept on file) □ Treated, potentially contaminated water (eg. Filtration, ultraviolet, reclaimed water): □ Treatment equipment is approved for food contact use □ The treatment system is operated, cleaned and maintained as per manufacturers instructions □ Sampled monthly while in operation □ Sampled from the source (outlet where you draw the water from). Refer to CFIA web site ‘Preventive Controls Food Businesses- Sampling procedures for how to draw the water □ The result must be = total coliforms undetectable in 100ml (kept on file) □ Describe Other Water Treatment/Supply Systems Used __________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________ | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) (Example: Seawater intake is next to waste discharge pipe on the vessel. Intake must be moved to front of the vessel. Scheduled on Maintenance/Repair Table) |
| --- | --- |

| PRE-SEASON WASTE DISPOSAL INSPECTION FORM (element 5.4) |  |  | Complete prior to the start of the season |  |
| --- | --- | --- | --- | --- |
| Monitor (signature): | Date Completed: | Verifier (signature): Check that all relevant inspection points have been completed |  | Date Verified: Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |
| Drainage and Sewage: □- must accommodate the volume and type of effluent produced, allowing for constant fluid removal with no pooling □- Drainage and sewage systems are equipped with traps and vents to prevent backflow □- Effluent or sewage lines do not pass directly over or through production areas □- Drainage and sewage systems located as far away from water intakes as possible, closer to the stern. □- Human waste not released within 3 nautical miles of the shore or where you source seawater □- Human waste receptacles are emptied into an approved sewage disposal system after returning to shore and then cleaned □- you remove and dispose of used and contaminated ice as soon as your catch is unloaded □- Trash disposed in land based waste disposal system Waste Disposal Waste Disposal systems must be (check as they are inspected): Check all that apply: □- Waste bins □- Scoops □- Shovels □- Bags □- Other (list all others): Ensure that: □- Waste control utensils and containers are labelled for waste use only □- provided with fitted lids □- of approved material □- properly maintained. | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Complete Corrective Actions if other than routine maintenance) (Example: Waste disposal shovels are not labelled for use and could be used to handle prawns. Corrective Action started) |  |

| PRE-SEASON PACKAGING CONTROLS INSPECTION FORM (element 1.2a&b) |  |  | Complete prior to the start of the season |
| --- | --- | --- | --- |
| Monitor (signature): | Date Monitored: | Verifier (signature): Date Verified: Review all points, review corrective actions, initiate new corrective actions if needed Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |  |
| All packaging material that comes into contact with commodities must be suitable to their intended use: Packaging material is sourced (check all that apply): □ From a single Processor □ Multiple Processors □ by the Vessel Operator |  |  |  |
| Types of Packaging Materials Used (element 1.2a) (check all that apply): - boxes- (made of smooth, non-absorbent, non-corrodible material, free from cracks and crevices, provide drainage and protect the fish from damage by crushing) - plastic films, - nettings, - trays - pouches. - other(s): __________________________________________ □ Ensure Each packaging type has an ‘Approval Document’ on file & readily available (see Packaging Controls- element 1.2a for details) Product ID-Labelling (element 1.2b) Pre-Stenciled packages must (check when completed): - Have a letter of guarantee (or a Supplier Food Safety Assurance Program) on file- indicating that labelling complies with SFCR: 92(1&2), 89(1). - Indicate SULFITES and PRAWNS/SHRIMP in the ingredient section - Compare ingredients on preservative label to chemical ingredients on packaging (follow up with CFIA or buyer (if they supplied packaging) if there are differences)- recheck if chemicals are changed during season (refer to 1.2b for example) - Indicate KEEP FROZEN on the packaging/label - Contain - common name - name and principal place of business of the person by or for whom the food was manufactured, prepared, produced, stored, packaged or labelled - Lot coding procedures supplied by the processor who will purchase the prawns (available for the crew) - Additional information that is required must be supplied by the processor in the form of a written document. Ensure this is available and followed _____________________________________________________________________________________________________ DEFICIENCY DESCRIPTION/COMMENTS (Initiate Complete Corrective Actions if needed) (Example: Packaging material ‘Approval Documents’ could not be located. Corrective Action initiated to get ‘letter of guarantee’ from the supplier. |  |  |  |

| PRE-SEASON CHEMICAL INSPECTION FORM (element 2.3 & 1.1b) |  |  | Complete prior to the start of the season |
| --- | --- | --- | --- |
| Monitor (signature): | Date Monitored: | Verifier (signature): Date Verified: Review all points, review corrective actions, initiate new corrective actions if needed Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |  |
| Pre-Season Chemical Controls: (Element 2.3) |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Make any corrective actions as needed) |
| Types of chemicals used (Check all that apply): □- Prawn Preservative (one formula only) □- Prawn Preservative (different formulae for different markets or processors) □- Detergent/Degreaser □- Bleach □- Sanitizer □- Pest Control chemicals □-Lubricants □- Other (list any additional chemicals used) __________________________________________________________________ __________________________________________________________________ __________________________________________________________________ __________________________________________________________________ Chemical Selection (ensure each control is completed): For each of the chemicals above, ensure: - Chemicals appropriately selected (see elements 2.3 & 1.1b) - Approval Letters on File Chemical Storage (check off as completed): - Prawn preservative stored away from non-food chemicals - stored in a designated, appropriate, weather proof location, clean and good repair, well ventilated - conditions must be as recommended by manufacturer (eg. temperature, humidity, light) if applicable - no unnecessary chemicals shall be stored in the processing area - not stored near supplies or materials (eg product, packaging, labels) that could contaminate food - the containers must be properly labelled, clearly identifying the product and its use. If the manufacturers labels are absent or inadequate, affix WHMIS compliant labels |  |  | (Example: Prawn Preservative stored with cleaning chemicals. Corrective Action initiated to inspect the preservative for contamination and move it to the correct storage area) |

| PRE-SEASON CONVEYANCE INSPECTION FORM (element 6.1a) |  |  | Complete prior to the start of the season |
| --- | --- | --- | --- |
| Monitor (signature): | Date Monitored: | Verifier (signature): Check that all relevant inspection points have been completed | Date Verified: Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |
| CONVEYANCES FOR TRANSPORT: (Element 6.1a) |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Take Corrective Actions if other than routine maintenance) |
| Conveyances used to transport to or from vessel must be: □- designed, constructed and maintained to prevent contamination of the food; □- constructed of, and maintained using, materials that are suitable for their intended use and, if the materials present a risk of contamination of the food, that are □- durable, □- capable of withstanding repeated cleaning and, if necessary to prevent contamination of a food, repeated sanitizing, and □- free of any noxious constituent; □- capable of maintaining the temperature and humidity at levels that are appropriate for the food and, if necessary to prevent contamination of the food, be equipped with instruments that control, indicate and record those levels; □-must not contain any animal, other than an animal referred to in paragraph SFCR 51(2)(a), any pest control product as defined in subsection 2(1) of the Pest Control Products Act or any other material or substance that presents a risk of contamination of the food; and □- clean and in a sanitary condition at the time of unloading or loading. |  |  | (Example: Door unable to fully close so temperature cannot be maintained and contamination can get in. Repair Scheduled on the Maintenance/Repair Table) |

| PRE-SEASON FREEZER AND COOLER STORAGE TEMPERATURE INSPECTION FORM (element 6.1b) Complete prior to the start of the season |  |  |  |
| --- | --- | --- | --- |
| Monitor (signature): | Date Monitored: | Verifier (signature): Check that all relevant inspection points have been completed | Date Verified: Ensure Deficiencies have been corrected. Initiate new corrective actions if needed |
| TEMPERATURE CONTROL AND STORAGE- equipment (Element 6.1b Conveyance & Storage Temperature & Humidity) List of freezer/cool storage containing conveyances (Check all that apply): □ vessel □ truck □ other:_________________________________ All temperature control systems must: - have equipment controls (able to set temperature) (SFCR 65(2)(a)) - Automatic temperature recording device installed or procedure to record temp manually every 24 hrs - records kept 3 years - be accessible for cleaning, maintenance or inspection, is able to be disassembled (SFCR 65(2)(b) ) - be able to can withstanding repeated cleaning; (SFCR 65(2)(c) ) - be made of approved materials (approval letter on file- see element 4.1 for selection method) - Calibrated as per manufacturers recommendations - 3RD party operated conveyances must be temperature checked at loading/unloading Freezer Facilities: □- Capable of freezing fish to -20°C (thermal core) within 36 hrs at a rate of 25mm/2hrs (test at least once per season- record results on this form or a ‘Processing Control Form’ in the During-Season Log if done during the season) □- Air current in blast freezer is -30°C; velocity of 125 m/min □- Brine freezer freeze fish in well-agitated brine solution to -15°C or colder □- adequate to prevent deterioration (-18C Max, -30C recommended) (SFCR 74) □- rooms maintained at proper temperature (-18C Max) (SFCR 74) □- equipped with accurate thermometer or temperature measuring device located in an area where the average air temperature in the room can be measured □- fish are moved to the freezers only after the freezing process is fully completed □- fluctuations in temperature are prevented because they can adversely affect the quality of frozen fish □- temperature is read and recorded at least once per day or continually with an automatic temperature recorder □- using shelving to prevent fish from getting crushed when stored in holds with a depth greater than 90 cm □-species of fish that are highly susceptible to crushing, such as crustaceans, mackerel, herring, and crab, should be stored at a maximum depth of 60 cm to protect them from damage which can cause spoilage and can expose them to contamination □- using chutes, flumes or other devices to slide the fresh fish down into the hold rather than throwing or dropping the fish into the hold □- storing the catch from different fishing days separately □- frost build-up minimal |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Take Corrective Actions if other than routine maintenance) (Example: Freezer cannot reach temperatures below -10°C. Technician contacted. Repair date scheduled on Maintenance/Repair Table) |
| Cooling Systems (fish cooling): - -made of stainless steel, high-density plastics, seawater resistant aluminum or copper-based alloys - -compressor capable of preventing significant rise in the temperature of the chilled water, or brine solution, when holding tanks filled with fish - capable of maintaining -1 to 4°C (SFCR 65(1) ) - - Calibrated regularly to ensure fish temperature between -1 & 2°C - -allows slush ice or cold water to circulate effectively around fish - -cooling system designed to facilitate the introduction and circulation of the cleaning and disinfecting solution throughout the system (SFCR 65(2)(b) ) - functions as intended (SFCR 65(2)(d) ) - prevent direct contact with the warmer surfaces of the vessel (SFCR 74) |  |  | DEFICIENCY DESCRIPTION/COMMENTS (Schedule repairs or replacement on Maintenance/Repair Table. Take Corrective Actions if other than routine maintenance) |

| Maintenance/Repair Table | Verification Date |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Verifier Initials |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Technician Initials |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Actual Repair Date |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Anticipated Repair Date |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Deficiency Noted |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Date |  |  |  |  |  |  |  |  |  |  |  |  |  |